GATEWICK COSTUME & PROP RENTAL HOUSE

Computer Network Design & Implementation Report

Cisco Packet Tracer Case Study

| Field | Details |
| --- | --- |
| Author | Sanika Kangane |
| Program | B.Tech CSE ’29 |

Prepared for college submission

# 1. Introduction

Gatewick Costume & Prop Rental House currently has no computer network. This case study proposes a structured IPv4 network for the Warehouse, Fitting Studio, Administration and IT departments, together with a point-to-point WAN connection to the offsite Prop Workshop. The design uses VLSM with 20% spare capacity, VLAN segmentation, IEEE 802.1Q trunking and Router-on-a-Stick inter-VLAN routing.

# 2. Objectives

Design an Extended Star topology.

Apply VLSM to 10.200.0.0/22 with 20% spare capacity.

Separate departments using VLANs 10, 20, 30 and 40.

Use access ports for end devices and trunk ports for uplinks.

Use IEEE 802.1Q and Router-on-a-Stick for inter-VLAN routing.

Provide a /30 point-to-point WAN link to the Prop Workshop.

Verify VLANs, trunks, router interfaces and connectivity.

# 3. Proposed Topology

Figure 1. Cisco Packet Tracer topology for the Gatewick network

Source: user-provided Cisco Packet Tracer screenshot. The figure is included as the actual topology reference for this report.

The recommended topology is an Extended Star. S0-Core is the central switch, department switches connect to it, R0-Main provides Router-on-a-Stick routing, and R0-Main connects to R1-Workshop through the point-to-point WAN.

| Technology | Purpose |
| --- | --- |
| VLANs | Logical separation of departments |
| Access ports | Connect end devices to their VLAN |
| Trunk ports | Carry multiple VLANs |
| IEEE 802.1Q | VLAN tagging on trunks |
| Router-on-a-Stick | Inter-VLAN routing using subinterfaces |
| VLSM | Efficient subnet allocation |
| Point-to-Point WAN | Remote workshop connectivity |
| IPv4 / ICMP / ARP | Addressing and connectivity testing |

# 4. Physical Connections

| From | Interface | To | Interface | Purpose |
| --- | --- | --- | --- | --- |
| R0-Main | G0/0/0 | S0-Core | Gi0/1 | Router-to-core trunk |
| R0-Main | G0/0/1 | R1-Workshop | G0/0/0 | WAN / leased line |
| S0-Core | Fa0/1 | S2-Fitting | Gi0/1 | Department uplink |
| S0-Core | Fa0/2 | S4-Warehouse | Gi0/1 | Department uplink |
| S0-Core | Gi0/2 | S1-Admin | Gi0/1 | Department uplink |
| S0-Core | Fa0/3 | S3-IT | Gi0/1 | Department uplink |

## PC Connections

| Switch | Connections |
| --- | --- |
| S4-Warehouse | Fa0/1 → PC0; Fa0/2 → PC1; Fa0/3 → PC2 |
| S2-Fitting | Fa0/1 → PC4; Fa0/2 → PC5 |
| S1-Admin | Fa0/1 → PC3; Fa0/2 → PC6 |
| S3-IT | Fa0/1 → PC7; Fa0/2 → PC8 |

# 5. VLSM and IP Addressing

Starting block: 10.200.0.0/22. The following VLSM plan follows the supplied host requirements and 20% spare-capacity calculations.

| Network | Hosts | 20% Capacity | Subnet | Mask | Usable Range | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Warehouse | 44 | 53 | 10.200.0.0/26 | 255.255.255.192 | 10.200.0.1–10.200.0.62 | 10.200.0.63 | 10.200.0.1 |
| Fitting Studio | 16 | 20 | 10.200.0.64/27 | 255.255.255.224 | 10.200.0.65–10.200.0.94 | 10.200.0.95 | 10.200.0.65 |
| Administration | 10 | 12 | 10.200.0.96/28 | 255.255.255.240 | 10.200.0.97–10.200.0.110 | 10.200.0.111 | 10.200.0.97 |
| IT | 6 | 8 | 10.200.0.112/28 | 255.255.255.240 | 10.200.0.113–10.200.0.126 | 10.200.0.127 | 10.200.0.113 |
| WAN | 2 | 2 | 10.200.0.128/30 | 255.255.255.252 | 10.200.0.129–10.200.0.130 | 10.200.0.131 | — |

Remaining spare address space: 10.200.0.132 – 10.200.3.255.

# 6. VLAN Design

| VLAN ID | Department | Network | Gateway |
| --- | --- | --- | --- |
| 10 | Warehouse | 10.200.0.0/26 | 10.200.0.1 |
| 20 | Fitting | 10.200.0.64/27 | 10.200.0.65 |
| 30 | Administration | 10.200.0.96/28 | 10.200.0.97 |
| 40 | IT | 10.200.0.112/28 | 10.200.0.113 |

Access ports: S4-Warehouse Fa0/1–Fa0/3 → VLAN 10; S2-Fitting Fa0/1–Fa0/2 → VLAN 20; S1-Admin Fa0/1–Fa0/2 → VLAN 30; S3-IT Fa0/1–Fa0/2 → VLAN 40. Department uplinks are trunks. S0-Core Gi0/1 connects to R0-Main as a trunk.

# 7. PC IP Addressing

| PC | Department | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| PC0 | Warehouse | 10.200.0.2 | 255.255.255.192 | 10.200.0.1 |
| PC1 | Warehouse | 10.200.0.3 | 255.255.255.192 | 10.200.0.1 |
| PC2 | Warehouse | 10.200.0.4 | 255.255.255.192 | 10.200.0.1 |
| PC4 | Fitting | 10.200.0.66 | 255.255.255.224 | 10.200.0.65 |
| PC5 | Fitting | 10.200.0.67 | 255.255.255.224 | 10.200.0.65 |
| PC3 | Administration | 10.200.0.98 | 255.255.255.240 | 10.200.0.97 |
| PC6 | Administration | 10.200.0.99 | 255.255.255.240 | 10.200.0.97 |
| PC7 | IT | 10.200.0.114 | 255.255.255.240 | 10.200.0.113 |
| PC8 | IT | 10.200.0.115 | 255.255.255.240 | 10.200.0.113 |

DNS: 0.0.0.0 / blank because no DNS server is configured.

# 8. Core Networking Concepts

| Concept | Simple Explanation |
| --- | --- |
| VLAN | A logical network segment that separates devices into different broadcast domains. |
| Access Port | A switch port assigned to one VLAN, normally used for an end device. |
| Trunk Port | A link carrying traffic for multiple VLANs. |
| IEEE 802.1Q | The standard used for VLAN tagging on trunk links. |
| Subinterface | A logical interface created under a physical router interface for a VLAN. |
| Router-on-a-Stick | Inter-VLAN routing using one physical router interface with multiple subinterfaces. |
| Inter-VLAN Routing | Routing traffic between different VLANs or IP subnets. |
| Default Gateway | The router address used to reach other networks. |
| VLSM | Variable Length Subnet Masking, which assigns subnet sizes according to host needs. |
| Subnet Mask | Identifies the network and host portions of an IPv4 address. |
| WAN | A network connection covering a larger geographic area. |
| Point-to-Point Connection | A direct link between two devices; /30 provides two usable IPv4 addresses here. |
| MAC Address | A Layer 2 address used for Ethernet frame delivery. |
| IP Address vs MAC Address | IP identifies an interface at Layer 3; MAC identifies an Ethernet interface at Layer 2. |
| ARP | Finds the MAC address associated with a local IPv4 address. |
| ICMP / Ping | ICMP provides control/diagnostic messages; ping uses ICMP Echo to test reachability. |
| DHCP | Can automatically provide IP configuration such as address, mask, gateway and DNS. |
| DNS | Translates names into IP addresses. |

# 9. Router-on-a-Stick Configuration

enable
configure terminal

interface gigabitEthernet 0/0/0
no ip address
no shutdown
exit

interface gigabitEthernet 0/0/0.10
encapsulation dot1Q 10
ip address 10.200.0.1 255.255.255.192
exit

interface gigabitEthernet 0/0/0.20
encapsulation dot1Q 20
ip address 10.200.0.65 255.255.255.224
exit

interface gigabitEthernet 0/0/0.30
encapsulation dot1Q 30
ip address 10.200.0.97 255.255.255.240
exit

interface gigabitEthernet 0/0/0.40
encapsulation dot1Q 40
ip address 10.200.0.113 255.255.255.240
exit

end

WAN addressing: R0-Main G0/0/1 = 10.200.0.129/30; R1-Workshop G0/0/0 = 10.200.0.130/30. R0 ↔ R1 ping was successfully verified with 100% success.

# 10. VLAN Verification

Figure 1. Reconstructed Verification Output — S0-Core show vlan brief. Not an actual Packet Tracer screenshot.

# 11. Trunk Verification

Figure 2. Reconstructed Verification Output — S0-Core show interfaces trunk. Not an actual Packet Tracer screenshot.

# 12. Router Interface Verification

Figure 3. Reconstructed Verification Output — R0-Main show ip interface brief. Not an actual Packet Tracer screenshot.

Figure 4. Reconstructed Verification Output — Router-on-a-Stick configuration. Not an actual Packet Tracer screenshot.

# 13. Connectivity Testing

Figure 5. Reconstructed Verification Output — PC0 → PC1. Not an actual Packet Tracer screenshot.

Figure 6. Reconstructed Verification Output — PC0 → Fitting Studio. Not an actual Packet Tracer screenshot.

Figure 7. Reconstructed Verification Output — PC0 → R1-Workshop. Not an actual Packet Tracer screenshot.

# 14. DHCP → ARP → DNS → ICMP

DHCP can provide a host with an IP address, subnet mask, default gateway and DNS server. ARP is then used when the host needs the MAC address associated with a local IPv4 address. DNS translates a domain or host name into an IP address. ICMP is used by tools such as ping to test whether a destination is reachable. For this actual Gatewick project, IP addresses were manually configured, DHCP was not configured, DNS was not configured, and ping/ICMP was used for connectivity testing. Therefore, DHCP and DNS are included as concepts, not as implemented services.

# 15. OSI / TCP-IP Activity Mapping

| Activity | Layer | Reason |
| --- | --- | --- |
| Barcode scanner updates inventory dashboard | Application layer | Application-level network service. |
| Switch forwards a frame using MAC address | Data Link layer | Ethernet frames and MAC addresses. |
| Faulty cable causes signal loss | Physical layer | Cables and signals operate at the physical layer. |
| Router chooses the best path | Network layer | Routing uses IP addresses. |
| Firewall tracks an open inventory-system session | Transport layer | TCP sessions and ports are transport-layer functions. |

# 16. Important Cisco Commands

| Command | Simple Explanation |
| --- | --- |
| enable | Enters privileged EXEC mode. |
| configure terminal | Enters global configuration mode. |
| exit | Moves back one configuration level. |
| end | Returns to privileged EXEC mode. |
| interface gigabitEthernet 0/0/0 | Selects the physical router interface. |
| no shutdown | Activates the interface. |
| no ip address | Removes an IP address from the interface. |
| interface g0/0/0.10 | Selects the VLAN 10 subinterface. |
| encapsulation dot1Q 10 | Associates the subinterface with VLAN 10 using 802.1Q. |
| ip address | Assigns an IPv4 address and subnet mask. |
| vlan 10 | Creates or selects VLAN 10. |
| name Warehouse | Names the VLAN Warehouse. |
| switchport mode access | Makes a switch port an access port. |
| switchport access vlan 10 | Places an access port into VLAN 10. |
| switchport mode trunk | Configures a switch port as a trunk. |
| show vlan brief | Displays VLANs, status and assigned access ports. |
| show interfaces trunk | Displays trunk status and VLAN information. |
| show ip interface brief | Summarizes interface IP addresses and status. |
| show mac address-table | Displays learned MAC addresses and ports. |
| ping | Tests reachability using ICMP Echo messages. |

# 17. Troubleshooting

During testing, one failed ping was caused by entering 100.200.0.3 instead of 10.200.0.3. The corrected command, ping 10.200.0.3, returned 4/4 replies with 0% packet loss. The issue was therefore an incorrect destination IP address rather than a network configuration failure.

# 18. Verification Summary

| Test | Expected Result | Result |
| --- | --- | --- |
| S0-Core VLANs | VLANs 10,20,30,40 active | Successful |
| Core/access uplinks | 802.1Q trunking | Successful |
| R0 subinterfaces | Correct gateway IPs and up/up | Successful |
| PC0 → PC1 | Same-VLAN communication | 4/4 replies, 0% loss |
| PC0 → PC4 | Inter-VLAN communication | 4/4 replies, 0% loss |
| PC0 → R1 | Remote WAN communication | 4/4 replies, 0% loss |

# 19. Conclusion

The Gatewick network design provides a practical structure for a company starting without an existing computer network. VLSM efficiently divides the 10.200.0.0/22 block while preserving spare capacity. VLANs separate departments logically, 802.1Q trunks carry multiple VLANs, Router-on-a-Stick enables inter-VLAN routing, and the /30 WAN subnet supports the two-router point-to-point connection. The supplied verification results show successful VLAN, trunk, interface, same-VLAN, inter-VLAN and WAN connectivity tests.

# 20. Viva Quick Reference

| Question | Short Answer |
| --- | --- |
| Why VLSM? | To efficiently use IP addresses by assigning different subnet sizes. |
| Why VLANs? | To logically separate departments. |
| Why trunk ports? | To carry multiple VLANs over one link. |
| Why Router-on-a-Stick? | To route between multiple VLANs using one physical router interface and multiple subinterfaces. |
| Why /30 for WAN? | It provides exactly two usable IPv4 addresses, suitable for a point-to-point link. |
| What is the default gateway? | The router address used to reach other networks. |
| What proves the network works? | Successful ping tests with 0% packet loss, including inter-VLAN and WAN tests. |

## Device Reference

| Type | Names |
| --- | --- |
| Routers | R0-Main; R1-Workshop |
| Switches | S0-Core; S4-Warehouse; S2-Fitting; S1-Admin; S3-IT |
| Warehouse PCs | PC0; PC1; PC2 |
| Fitting PCs | PC4; PC5 |
| Administration PCs | PC3; PC6 |
| IT PCs | PC7; PC8 |

# 21. Cisco Command Prompt Verification

The following pages present Cisco Packet Tracer-style command prompt views used to verify the VLANs, trunk links, router subinterfaces, IP addressing, and connectivity of the Gatewick network. They are formatted as command-prompt verification pages for clear documentation.

## 21.1 S0-Core - Cisco IOS CLI

Cisco Packet Tracer-style reconstructed command prompt for report documentation

## 21.2 R0-Main - Cisco IOS CLI

Cisco Packet Tracer-style reconstructed command prompt for report documentation

## 21.3 PC0 - Packet Tracer Command Prompt

Cisco Packet Tracer-style reconstructed command prompt for report documentation

## 21.4 R0-Main - WAN Verification

Cisco Packet Tracer-style reconstructed command prompt for report documentation